Processor: Datapath

© IT - TDT	Computer Organisation

# Acknowledgement

- The contents of these slides have origin from School of Computing, National University of Singapore.

- We greatly appreciate support from Mr. Aaron Tan Tuck Choy for kindly sharing these materials.

© IT - TDT	Computer Organisation

# Policies for students

- These contents are only used for students PERSONALLY.

- Students are NOT allowed to modify or deliver these contents to anywhere or anyone for any purpose.

# Road Map: Part II

n Processor Datapath q Generic Execution Stages q MIPS Execution Stages q Constructing Datapath

Assembly

Language

Processor:

Datapath

Processor:

Control

Pipelining

Cache

Assembly

Language

Processor:

Datapath

Processor:

Control

Pipelining

Cache

Performance

Building a Processor: Datapath & Control

- Two major components for a processor:

- Datapath

- Collection of components that process data

- Performs the arithmetic, logical and memory operations

- Control

- Tells the datapath, memory, and I/O devices what to do according to program instructions

# MIPS Processor: Implementation

- Simplest possible implementation of a subset of the core MIPS ISA:

- Arithmetic and Logical operations

- add, sub, and, or, addi, andi, ori,  slt

- Data transfer instructions

- lw, sw

- Branches

- beq, bne

- Shift instructions (sll, srl) and J-type instructions (j) will not be discussed here

- Left as exercises J

Instruction

Fetch

Instruction

Decode

Operand

Fetch

Execute

Result

Write

Next Instruction

Instruction

Fetch

Instruction

Decode

Operand

Fetch

Execute

Result

Write

Next Instruction

Recap: Instruction Execution Cycle

- Fetch:

- Get instruction from memory

- Address is in Program Counter (PC) Register

- Decode:

- Find out the operation required

- Operand Fetch:

- Get operand(s) needed for operation

- Execute:

- Perform the required operation • Result Write (WriteBack):

- Store the result of the operation

# MIPS Instruction Executions

- Show the actual steps for 3 representative MIPS instructions

- Fetch and Decode stages not shown: • The standard steps are performed

|  |  | lwlw$3$3,, 2020( ($1 )) | beqbeq$1$1, , $2$2, , labelofst |
| --- | --- | --- | --- |
|  | add $3, $1, $2 |  |  |
| Fetch | standard | standard | standard |
| Decode |  |  |  |
| Operand Fetch | o Read [$1] as opr1 o Read [$2] as opr2 | o Read [$1] as opr1 o Use 20 as opr2 | o Read [$1] as opr1 o Read [$2] as opr2 |
| Execute | Result = opr1 + opr2 | o MemAddr = opr1 + opr2 o Use MemAddr to read data from memory | Taken = (opr1 == opr2 )? Target = (PC+4) or (PC+4) + ofst |
| Result Write | Result stored in $3 | Memory data stored in $3 | PC = Target |

- opr = Operand	n ofst = offset

- MemAddr = Memory Address

# 5-STAGE MIPS EXECUTION

- Design changes:

- Merge Decode and Operand Fetch – Decode is simple for MIPS

- Split Execute into ALU (Calculation) and Memory Access

|  |  |  | lwlw$3$3,, 2020( ($1 $1)) | beqbeq$1$1, , $2$2, , labelofst |
| --- | --- | --- | --- | --- |
|  | addadd $3$3, , $1$1, , $2$2 |  |  |  |
| Fetch | Read inst. at [PC] |  | Read inst. at [PC] | Read inst. at [PC] |
| Decode & Operand Fetch | o Read [$1] as opr1 o Read [$2] as opr2 |  | o Read [$1] as opr1 o Use 20 as opr2 | o Read [$1] as opr1 o Read [$2] as opr2 |
| ALU | Result = opr1 + opr2 |  | MemAddr = opr1 + opr2 | Taken = (opr1 == opr2 )? Target = (PC+4) or (PC+4) + ofst |
| Memory Access |  |  | Use MemAddr to read datefrom memory |  |
| Result Write | Result stored in $3 |  | Memory data stored in $3 | PC = Target |

Let’s Build a MIPS Processor!

- What we are going to do:

- Look at each stage closely, figure out the requirements and processes

- Sketch a high level block diagram, then zoom in for each elements

- With the simple starting design, check whether different type of instructions can be handled:

- Add modifications when needed è Study the design from the viewpoint of a designer, instead of a “tourist” J

# Fetch Stage: Requirements

- Instruction Fetch Stage:

1. Use the Program Counter (PC) to fetch the instruction from memory

- PC is implemented as a special register in the processor

2. Increment the PC by 4 to get the address of the next instruction:

- How do we know the next instruction is at PC+4?

- Note the exception when branch/jump instruction is executed

- Output to the next stage (Decode):

- The instruction to be executed

Fetch Stage

:

Block Diagram

Add

PC

4

Read

address

Instruction

Instruction

memory

A register

Memory which

stores

program

instructions

A simple

adder

Decode Stage

Instruction

Fetch Stage

:

Block Diagram

Add

PC

4

Read

address

Instruction

Instruction

memory

A register

Memory which

stores

program

instructions

A simple

adder

Decode Stage

Instruction

# Element: Instruction Memory

- Storage element for the instructions

Instruction

Memory

Instruction

Address

Instruction

Instruction

Memory

Instruction

Address

Instruction

Recall: sequential circuit

- Has an internal state that stores information

- Clock signal is assumed and not shown

- Supply instructions given the address

- Given instruction address M as input, the 	Memory memory outputs the content at address M	………..

- Conceptual diagram of the memory layout is 	2048 add $3, $1, $2 given on the right è	2052 sll $4, $3, 2

2056 andi $1, $4, 0xF

……	………..

## Element: Adder

- Combinational logic to implement  the addition of two numbers

Sum

Add

A

B

A+B

Sum

Add

A

B

A+B

Inputs:

- Two 32-bit numbers A, B • Output:

- Sum of the input numbers, A + B

- Just a 32-bit version of the adder discussed in first part of the course J

## The Idea of Clocking

- It seems that we are reading and updating the PC at the same time: • How can it works properly?

- Magic of clock:

- PC is read during the first half of the clock period and it is updated with PC+4 at the next rising clock edge

Add

PC

4

Read

address

Instruction

Instruction

memory

In

Clk

Time

PC

100

104

108

112

In

104

108

112

116

Add

PC

4

Read

address

Instruction

Instruction

memory

In

Clk

Time

PC

100

104

108

112

In

104

108

112

116

## Decode Stage: Requirement

- Instruction Decode Stage:

- Gather data from the instruction fields:

  - Read the opcode to determine instruction type and field lengths

  - Read data from all necessary registers

- Can be two (e.g. add), one (e.g. addi) or zero (e.g. j)

- Input from previous stage (Fetch):

- Instruction to be executed

- Output to the next stage (ALU):

- Operation and the necessary operands

## Decode Stage: Block Diagram

Fetch Stage

Inst.

Read

register 1

Read

register 2

Write

register

Read

data 1

Read

data 2

Data

Register

Number

Register

File

5

5

5

Execute Stage

Operands

Collection of

registers, known

as

register file

Fetch Stage

Inst.

Read

register 1

Read

register 2

Write

register

Read

data 1

Read

data 2

Data

Register

Number

Register

File

5

5

5

Execute Stage

Operands

Collection of

registers, known

as

register file

# Element: Register File

- A collection of 32 registers:

- Each is 32-bit wide and can be read/written by specifying register number

- Read at most two registers per instruction

- Write at most one register per instruction

- RegWrite is a control signal to indicate:

- Writing of register

- 1(True) = Write,  0 (False) = No Write

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Data

Data

Register

Number

Register

File

5

5

5

32

32

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Data

Data

Register

Number

Register

File

5

5

5

32

32

RegWrite

## Decode Stage: R-Type Instruction

opcode

31:26

rs

25:21

rt

20:16

rd

15:11

shamt

10:6

funct

5:0

000000

01001

01010

01000

00000

100000

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

RegWrite

Register

File

5

5

5

content of

register

$9

content of

register

$10

Result to be

stored

(

produced by later

stage)

Inst [25:21]

Inst [20:16]

Inst [15:11]

Notation:

Inst [Y:X]

=

bits X to Y in Instruction

32

32

add $8, $9, $10

## Decode Stage: I-Type Instruction

addi $21, $22, -50

001000

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

RegWrite

Register

File

5

5

5

Content of

register

$22

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

Problems:

-

Destination

$21

is in the "wrong position"

-

Read Data 2

is an immediate value, not from register

32

32

001000

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

RegWrite

Register

File

5

5

5

Content of

register

$22

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

Problems:

-

Destination

$21

is in the "wrong position"

-

Read Data 2

is an immediate value, not from register

32

32

001000

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Register

File

5

5

5

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

RegDst

:

A control signal to choose

either Inst[20:16] or Inst[15:11]

as the write register number

Solution (

Wr

. Reg. No.

):

Use a

multiplexer

to choose the

correct write register number

based on instruction type

32

32

001000

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Register

File

5

5

5

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

RegDst

:

A control signal to choose

either Inst[20:16] or Inst[15:11]

as the write register number

Solution (

Wr

. Reg. No.

):

Use a

multiplexer

to choose the

correct write register number

based on instruction type

32

32

Decode Stage: Choice in Destination addi $21, $22, -50

## Recap: Multiplexer

- Function:

in

0

in

n

-

1

out

m

.

.

.

M

U

X

in

0

in

n

-

1

out

m

.

.

.

M

U

X

Selects one input from multiple input lines	Control

- Inputs: • n lines of same width

- Control: • m bits where n = 2m

- Output:

- Select ith input line if control=i Control=0 à select in0

Control=3 à select in3

## Decode Stage: Choice in Data 2 addi $21, $22, -50

001000

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

5

5

5

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Register

File

ALUSrc

:

A control signal to

choose either

"Read data 2" or

the sign extended

Inst[15:0] as the

second operand

Solution (

Rd. Data 2

):

Use a

multiplexer

to choose the correct operand 2.

Sign extend the 16

-

bit immediate value to 32

-

bit

32

32

001000

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

5

5

5

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Register

File

ALUSrc

:

A control signal to

choose either

"Read data 2" or

the sign extended

Inst[15:0] as the

second operand

Solution (

Rd. Data 2

):

Use a

multiplexer

to choose the correct operand 2.

Sign extend the 16

-

bit immediate value to 32

-

bit

32

32

Decode Stage: Load Word Instruction

- Try it out: "lw $21, -50($22)"

- Do we need any modification?

100011

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

5

5

5

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Register

File

32

32

100011

10110

10101

1111 1111 1100 1110

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

5

5

5

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Register

File

32

32

## Decode Stage: Branch Instruction

•

We will tackle this problem in the ALU Stage

J

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Register

File

32

32

•

We will tackle this problem in the ALU Stage

J

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Register

File

32

32

Example: "beq $9, $0, 3"

- Need to calculate branch outcome and target at the same time!

## Decode Stage: Summary

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Registers

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Operand 1

Operand 2

Inst[31:0]

32

32

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Registers

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

Operand 1

Operand 2

Inst[31:0]

32

32

### ALU Stage: Requirement

- Instruction ALU Stage:

- ALU = Arithmetic-Logic Unit

- Perform the real work for most instructions here

- Arithmetic (e.g. add, sub), Shifting (e.g. sll), Logical (e.g. and, or)

- Memory operation (e.g. lw, sw): Address calculation

- Branch operation (e.g. bne, beq): Perform register comparison and target address calculation

- Input from previous stage (Decode):

- Operation and Operand(s)

- Output to the next stage (Memory):

- Calculation result

### ALU Stage: Block Diagram

ALU

result

ALU

Decode Stage

Operands

Memory Stage

Logic to perform

arithmetic and

logical operations

ALU

result

ALU

Decode Stage

Operands

Memory Stage

Logic to perform

arithmetic and

logical operations

### Element: Arithmetic Logical Unit

ALU

result

ALU

ALUcontrol

4

isZero

?

A

B

A op B

(

A op B) == 0?

ALU

result

ALU

ALUcontrol

4

isZero

?

A

B

A op B

(

A op B) == 0?

ALU (Arithmetic-logical unit)

- Combinational logic to implement arithmetic and logical operations

- Inputs:

- Two 32-bit numbers

| ALUcontrol | Function |
| --- | --- |
| 0000 | AND |
| 0001 | OR |
| 0010 | add |
| 0110 | subtract |
| 0111 | slt |
| 1100 | NOR |

- Control:

- 4-bit to decide the particular operation

- Outputs:

- Result of arithmetic/logical operation

- A 1-bit signal to indicate whether result is zero

ALU Stage: Non-Branch Instructions

- We can handle non-branch instructions easily:

#### add $8, $9, $10

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Register

File

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

isZero

?

opcode

31:26

rs

25:21

rt

20:16

rd

15:11

shamt

10:6

funct

5:0

000000

01001

01010

01000

00000

100000

ALUcontrol

:

Set using

opcode

+

funct

field (more in next lecture)

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Register

File

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

isZero

?

opcode

31:26

rs

25:21

rt

20:16

rd

15:11

shamt

10:6

funct

5:0

000000

01001

01010

01000

00000

100000

ALUcontrol

:

Set using

opcode

+

funct

field (more in next lecture)

## ALU Stage: Brach Instructions

- Branch instruction is harder as we need to perform two calculations:

- Example: "beq $9, $0, 3"

1.	Branch Outcome:

- Use ALU unit to compare the register

- check (how?)

2.	Branch Target Address:

  - Introduce additional logic to calculate the address

  - Need PC (from Fetch Stage)

  - Need Offset (from Decode Stage)

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Register

File

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

isZero

?

Left Shift

2

-

bit

PC

Add

4

Add

M

U

X

PCSrc

PCSrc

:

Control Signal

to select

between

PC+4) or

(

Branch Target

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Read

register 1

Read

register 2

Write

register

Write

data

Read

data 1

Read

data 2

Register

File

5

5

5

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

isZero

?

Left Shift

2

-

bit

PC

Add

4

Add

M

U

X

PCSrc

PCSrc

:

Control Signal

to select

between

PC+4) or

(

Branch Target

Complete ALU Stage

E.g. "beq $9, $0, 3"

### Memory Stage: Requirement

- Instruction Memory Access Stage:

- Only the load and store instructions need to perform operation in this stage:

- Use memory address calculated by ALU Stage

- Read from or write to data memory

- All other instructions remain idle

- Result from ALU Stage will pass through to be used in Result Store (Writeback) stage if applicable

- Input from previous stage (ALU):

- Computation result to be used as memory address (if applicable) • Output to the next stage (Writeback):

- Result to be stored (if applicable)

### Memory Stage: Block Diagram

ALU Stage

Result

Result Store

Stage

Memory which

stores data

values

Data

Memory

Address

Read

Data

Write

Data

MemRead

MemWrite

ALU Stage

Result

Result Store

Stage

Memory which

stores data

values

Data

Memory

Address

Read

Data

Write

Data

MemRead

MemWrite

### Element: Data Memory

- Storage element for the data of a program

- Inputs: 	MemWrite

Data

Memory

Address

Read

Data

Write

Data

Data

Memory

Address

Read

Data

Write

Data

Memory Address

- Data to be written (Write Data) for store instructions

- Control:

- Read and Write controls; only one can be asserted at any point of time

- Output: 	MemRead

- Data read from memory (Read Data) for load instructions

### Memory Stage: Load Instructions

• Only relevant parts of Decode & ALU Stages are shown

#### lw $21, -50($22)

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

100011

10110

10101

1111 1111 1100 1110

Address

Write

Data

MemRead

MemWrite

Data

Memory

Read

Data

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

ALUSrc

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

100011

10110

10101

1111 1111 1100 1110

Address

Write

Data

MemRead

MemWrite

Data

Memory

Read

Data

### Memory Stage: Store Instructions

• Need Read Data 2 (Decode) as the Write Data

#### sw $21, -50($22)

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

101011

10110

10101

1111 1111 1100 1110

Address

Write

Data

MemRead

MemWrite

Data

Memory

Read

Data

000100

01001

00000

0000 0000 0000 0011

Immediate

15:0

opcode

31:26

rs

25:21

rt

20:16

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

101011

10110

10101

1111 1111 1100 1110

Address

Write

Data

MemRead

MemWrite

Data

Memory

Read

Data

#### Memory Stage: Non-Memory Instructions

• Add a multiplexer to choose the result to be stored

##### add $8, $9, $10

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

Data

Memory

Address

Read

Data

Write

Data

MemWrite

opcode

31:26

rs

25:21

rt

20:16

rd

15:11

shamt

10:6

funct

5:0

000000

01001

01010

01000

00000

100000

M

U

X

MemToReg

MemToReg

:

A control signal to indicate

whether result came from

memory or ALU unit

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

RegDst

Inst [

15:0]

M

U

X

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

16

32

ALU

result

ALU

ALUcontrol

4

Data

Memory

Address

Read

Data

Write

Data

MemWrite

opcode

31:26

rs

25:21

rt

20:16

rd

15:11

shamt

10:6

funct

5:0

000000

01001

01010

01000

00000

100000

M

U

X

MemToReg

MemToReg

:

A control signal to indicate

whether result came from

memory or ALU unit

### Result Write Stage: Requirement

- Instruction Register Write Stage:

- Most instructions write the result of some computation into a register

- Examples: arithmetic, logical, shifts, loads, set-less-than • Need destination register number and computation result • Exceptions are stores, branches, jumps:

- There are no results to be written èThese instructions remain idle in this stage

- Input from previous stage (Memory):

- Computation result either from memory or ALU

### Result Write Stage: Block Diagram

Memory Stage

Result

Read

register 1

Read

register 2

Write

register

Read

data 1

Read

data 2

Registers

5

5

5

Write

data

Memory Stage

Result

Read

register 1

Read

register 2

Write

register

Read

data 1

Read

data 2

Registers

5

5

5

Write

data

- Result Write stage has no additional element:

- Basically just route the correct result into register file

- The Write Register number is generated way back in the Decode Stage

FetchProcessor: DecodeDatapath

### Result Write Stage: Routing add $8, $9, $10

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

Inst [

15:0]

M

U

X

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

ALU

result

ALU

ALUcontrol

4

Data

Memory

Address

Read

Data

Write

Data

MemWrite

opcode

31:26

rs

25:21

rt

20:16

rd

15:11

shamt

10:6

funct

5:0

000000

01001

01010

01000

00000

100000

M

U

X

MemToReg

Inst [25:21]

Inst [20:16]

Inst [15:11]

M

U

X

Inst [

15:0]

M

U

X

RR1

RR2

WR

WD

RD1

RD2

Registers

5

5

5

RegWrite

Sign

Extend

ALU

result

ALU

ALUcontrol

4

Data

Memory

Address

Read

Data

Write

Data

MemWrite

opcode

31:26

rs

25:21

rt

20:16

rd

15:11

shamt

10:6

funct

5:0

000000

01001

01010

01000

00000

100000

M

U

X

MemToReg

The Complete Datapath!

- We have just finished “designing” the datapath for a subset of MIPS instructions:

- Shifting and Jumping are not supported

- Check your understanding:

- Take the complete datapath and play the role of controller:

- See how supported instructions are executed

- Figure out the correct control signals for the datapath elements

- Coming up next: Control (Lecture #16)

#### Datapath: Generic Steps

PC

instruction

memory

+4

rt

rs

rd

registers

ALU

Data

memory

imm

1

. Instruction

Fetch

2

. Decode/

Register

3

.

ALU

4

. Memory

5

.

Write

Back

Read

Read

Write

PC

instruction

memory

+4

rt

rs

rd

registers

ALU

Data

memory

imm

1

. Instruction

Fetch

2

. Decode/

Register

3

.

ALU

4

. Memory

5

.

Write

Back

Read

Read

Write

Read

#### Datapath Walkthroughs: ADD (1/2)

- add $r3,$r1,$r2 # r3 = r1+r2

- Stage 1: Fetch this instruction, increment PC.

- Stage 2: Decode to find that it is an add instruction, then read registers $r1 and $r2.

- Stage 3: Add the two values retrieved in stage 2.

- Stage 4: Idle (nothing to write to memory).

- Stage 5: Write result of stage 3 into register $r3.

#### Datapath Walkthroughs: ADD (2/2)

add $r3, $r1, $r2

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

2

1

3

add r3, r1, r2

reg[1]+reg[2]

reg[2]

reg[1]

Read

Read

Write

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

2

1

3

add r3, r1, r2

reg[1]+reg[2]

reg[2]

reg[1]

Read

Read

Write

#### Datapath Walkthroughs: SLTI (1/2)

- slti $r3,$r1,17

- Stage 1: Fetch this instruction, increment PC.

- Stage 2: Decode to find it is an slti, then read register $r1. § Stage 3: Compare value retrieved in stage 2 with the integer 17.

- Stage 4: Go idle.

- Stage 5: Write the result of stage 3 in register $r3.

#### Datapath Walkthroughs: SLTI (2/2)

slti $r3, $r1, 17

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

x

1

3

slti r3, r1, 17

reg[1]

-

17

17

reg[1]

Read

Read

Write

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

x

1

3

slti r3, r1, 17

reg[1]

-

17

17

reg[1]

Read

Read

Write

#### Datapath Walkthroughs: SW (1/2)

- sw $r3, 20($r1)

- Stage 1: Fetch this instruction, increment PC.

- Stage 2: Decode to find it is an sw, then read registers $r1 and $r3.

- Stage 3: Add 20 to value in register $r1 (retrieved in stage 2).

- Stage 4: Write value in register $r3 (retrieved in stage 2) into memory address computed in stage 3.

- Stage 5: Go idle (nothing to write into a register).

#### Datapath Walkthroughs: SW (2/2)

sw $r3, 20($r1)

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

3

1

x

sw r3, 20(r1)

reg[1]+20

20

reg[1]

MEM[r1+20]<

-

r3

reg[3]

Read

Read

Write

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

3

1

x

sw r3, 20(r1)

reg[1]+20

20

reg[1]

MEM[r1+20]<

-

r3

reg[3]

Read

Read

Write

Why Five Stages?

- Could we have a different number of stages? § Yes, and other architectures do.

- So why does MIPS have five stages, if instructions tend to go idle for at least one stage?

- There is one instruction that uses all five stages:

the load.

#### Datapath Walkthroughs: LW (1/2)

- lw $r3, 40($r1)

- Stage 1: Fetch this instruction, increment PC.

- Stage 2: Decode to find it is a lw, then read register $r1.

- Stage 3: Add 40 to value in register $r1 (retrieved in stage 2).

- Stage 4: Read value from memory address compute in stage 3.

- Stage 5: Write value found in stage 4 into register $r3.

#### Datapath Walkthroughs: LW (2/2)

lw $r3, 40($r1)

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

x

1

3

lw r3, 40(r1)

reg[1]+40

r3<

-

MEM[r1+40]

reg[3]

Read

Read

Write

40

reg[1]

PC

instruction

memory

+4

registers

ALU

Data

memory

imm

x

1

3

lw r3, 40(r1)

reg[1]+40

r3<

-

MEM[r1+40]

reg[3]

Read

Read

Write

40

reg[1]

What Hardware is Needed?

- PC: a register which keeps track of address of the next instruction. § General Purpose Registers

- Used in stage 2 (read registers) and stage 5 (writeback).

- Memory

- Used in stage 1 (fetch) and stage 4 (memory).

- Cache system makes these two stages as fast as the others, on average.

#### Datapath: Summary

- Construct datapath based on register transfers required to perform instructions.

- Control part causes the right transfers to happen.

PC

instruction

memory

+4

rt

rs

rd

registers

ALU

Data

memory

imm

Controller

opcode, funct

PC

instruction

memory

+4

rt

rs

rd

registers

ALU

Data

memory

imm

Controller

opcode, funct

#### Reading Assignment

- The Processor: Datapath and Control

- 3rd edition: Chapter 5 Sections 5.1 – 5.3

- 4th edition: Chapter 4 Sections 4.1 – 4.3

© IT - TDT	Computer Organisation

# Q&A