Chương 4: Karnaugh-maps:

* Sử dụng Karnaugh maps rút gọn biểu thức với 3 bit.

The answers:

3.1

- F(x,y,z) = (0,2,4,5)= x’z’ + xy’

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 |  |  | 1 |
| 1 | 1 | 1 |  |  |

- F(x,y,z) = (0,2,4,5,6) = z’ + xy’

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 |  |  | 1 |
| 1 | 1 | 1 |  | 1 |

- F(x,y,z) = (0,1,2,3,5)= x’ + y’z

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 |
| 1 |  | 1 |  |  |

- F(x,y,z) = (1,2,3,7)=x’z+x’y+yz

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  | 1 | 1 | 1 |
| 1 |  |  | 1 |  |

3.2

- ( F(x,y,z) = (0,1,5,7)=x’y’ + xz

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  |  |
| 1 |  | 1 | 1 |  |

- F(x,y,z) = (1,2,3,6,7)=y+x’z

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  | 1 | 1 | 1 |
| 1 |  |  | 1 | 1 |

- F(x,y,z) = (2,3,4,5)= x’y + xy’

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  |  | 1 | 1 |
| 1 | 1 | 1 |  |  |

- F(x,y,z) = (1,2,3,5,6,7)= z+y

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  | 1 | 1 | 1 |
| 1 |  | 1 | 1 | 1 |

- F(x,y,z) = (0,2,4,6)= x’y’ + xz’

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  |  |
| 1 | 1 |  |  | 1 |

- F(x,y,z) = (3,4,5,6,7)= x+yz

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  |  | 1 |  |
| 1 | 1 | 1 | 1 | 1 |

3.3

- xy+x’y’z’+x’yz’

= xyz+xyz’+x’y’z’+x’yz’= x’z’+xy

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 |  |  | 1 |
| 1 |  |  | 1 | 1 |

- x’y’+yz+x’yz’

= x’y’z+x’y’z’+xyz+x’yz+x’yz’= z+x’y’

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |  |
| 1 |  | 1 | 1 |  |

- xy’+yz’+y’z’

= x’yz+x’yz’+xyz’+xy’z’+x’y’z’= y’z’+xy’+xz’+xyz

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 1 |  | 1 |  |
| 1 | 1 | 1 |  | 1 |

- X’yz+xy’z’+xy’z = xy’ + x’yz

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  |  | 1 |  |
| 1 | 1 | 1 |  |  |

3.4

- F(x,y,z)= (2,3,6,7)=x’z+xy

| X YZ | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  | 1 | 1 |  |
| 1 |  |  | 1 | 1 |

- F(A,B,C,D)=(4,6,7,15)=A’BD’+BCD

| CD AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  | 1 |  |  |
| 01 |  |  |  |  |
| 11 |  | 1 | 1 |  |
| 10 |  | 1 |  |  |

- F(A,B,C,D)=(3,7,11,13,14,15)= CD+ABD+ABC

| CD AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  |  |  |
| 01 |  |  | 1 |  |
| 11 | 1 | 1 | 1 | 1 |
| 10 |  |  | 1 |  |

- F (w,x,y,z)=(2,3,12,13,14,15)=wx+w’x’y

| yz wx | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  | 1 |  |
| 01 |  |  | 1 |  |
| 11 | 1 |  | 1 |  |
| 10 | 1 |  | 1 |  |

- F (w,x,y,z)=(11,12,13,14,15)=WX+WYZ

| YZ WX | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  | 1 |  |
| 01 |  |  | 1 |  |
| 11 |  |  | 1 | 1 |
| 10 |  |  | 1 |  |

- F (w,x,y,z)=(8,10,12,13,14)=WZ’+WXY’

| YZ WX | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  | 1 | 1 |
| 01 |  |  | 1 |  |
| 11 |  |  |  |  |
| 10 |  |  | 1 | 1 |

3.5

- F (w,x,y,z)=(1,4,5,6,12,14,15)= xz’+w’y’z+wxy

| YZ WX | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  | 1 | 1 |  |
| 01 | 1 | 1 |  |  |
| 11 |  |  | 1 |  |
| 10 |  | 1 | 1 |  |

- F (A,B,C,D)=(2,3,6,7,12,13,14)= a’c+abc’+bcd’

| CD AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  | 1 |  |
| 01 |  |  | 1 |  |
| 11 | 1 | 1 |  |  |
| 10 | 1 | 1 | 1 |  |

- F (w,x,y,z)=(1,3,4,5,6,7,9,11,13,15)= z+w’x

| YZ WX | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  | 1 |  |  |
| 01 | 1 | 1 | 1 | 1 |
| 11 | 1 | 1 | 1 | 1 |
| 10 |  | 1 |  |  |

- F (A,B,C,D)=(0,2,4,5,6,7,8,10,13,15)= b’d’+a’b+bd

| CD AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 1 |  | 1 |
| 01 |  | 1 | 1 |  |
| 11 |  | 1 | 1 |  |
| 10 | 1 | 1 |  | 1 |

3.6

- a’b’c’d’+ac’d’+b’cd’+a’bcd+bc’d

= a’b’c’d’ + abc’d’ + ab’c’d’+ab’cd’+a’b’cd’ + a’bcd + abc’d+a’bc’d

= b’d’+abc’+a’bd

| CD AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 |  | 1 | 1 |
| 01 |  | 1 | 1 |  |
| 11 |  | 1 |  |  |
| 10 | 1 |  |  | 1 |

- X’z+w’xy’+w(x’y+xy’)

= wx’yz+wx’y’z+w’x’yz+w’x’y’z+w’xy’z+wx’yz+wx’yz’+wxy’z+wxy’z’

= y’z+w’x’y’+w’x’z+wxy’+wx’y

| YZ WX | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 |  | 1 |  |
| 01 | 1 | 1 | 1 | 1 |
| 11 | 1 |  |  | 1 |
| 10 |  |  |  | 1 |

- A’B’C’D+AB’D+A’BC’+ABCD + AB’C

= A’B’C’D + AB’CD+AB’C’D+A’BC’D+A’BC’D’+ABCD+AB’CD’

=A’BC’+B’C’D+ACD+AB’C

| CD AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  | 1 |  |  |
| 01 | 1 | 1 |  | 1 |
| 11 |  |  | 1 | 1 |
| 10 |  |  |  | 1 |

- A’B’C’D’ + BC’D + A’C’D + A’BCD + ACD’

= A’B’C’D’ + ABC’D+A’BC’D+A’B’C’D+A’BCD+ABCD’+AB’CD’

=A’B’C’+A’BD+BC’D+ACD’

| CD AB | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 |  |  |  |
| 01 | 1 | 1 | 1 |  |
| 11 |  | 1 |  |  |
| 10 |  |  | 1 | 1 |

*Rút gọn biểu thức

* Sử dụng Karnaugh maps rút gọn biểu thức với 4 bit.

3.7 Simplify the following Boolean expressions, using four-variable maps:

(a) w’z + xz + x’y + wx’z

(b) AD’ + B’C’D + BCD’ + BC’D

(c) AB’C + B’C’D’ + BCD + ACD’ + A’B’C + A’BC’D

(d) wxy + xz + wx’z + w’x

* Từ F suy ra minterms và vẽ karnaugh maps

3.8 Find the minterms of the following Boolean expressions by first plotting each function in a map:

(a) xy + yz + xy’z

(b) C’D + ABC’ + ABD’ + A’B’D

(c) wyz + w’x’ + wxz’

(d) A’B + A’CD + B’CD + BC’D’

* Từ F tìm PIs và tìm EPIs:

3.9 Find all the prime implicants for the following Boolean functions, and determine which are essential:

(a) F (w, x, y, z) = ∑ (0, 2, 4, 5, 6, 7, 8, 10, 13, 15)

(b) F (A, B, C, D) = ∑ (0, 2, 3, 5, 7, 8, 10, 11, 14, 15)

(c) F (A, B, C, D) = ∑ (2, 3, 4, 5, 6, 7, 9, 11, 12, 13)

(d) F(w, x, y, z) = ∑ (1, 3, 6, 7, 8, 9, 12, 13, 14, 15)

(e) F(A, B, C, D) =∑ (0, 1, 2, 5, 7, 8, 9, 10, 13, 15)

(f) F(w, x, y, z) = ∑(0, 1, 2, 5, 7, 8, 10, 15)

* Rút gọn biểu thức F và tìm EPIs:

3.10 Simplify the following Boolean functions by first finding the essential prime implicants:

(a) F(w, x, y, z) = ∑(0, 2, 5, 7, 8, 10, 12, 13, 14, 15)

(b) F(A, B, C, D) = ∑ (0, 2, 3, 5, 7, 8, 10, 11, 14, 15)

(c) F(A, B, C, D) = ∑ (1, 3, 4, 5, 10, 11, 12, 13, 14, 15)

(d) F(w, x, y, z) = ∑(0, 1, 4, 5, 6, 7, 9, 11, 14, 15)

(e) F(A, B, C, D) = ∑(0, 1, 3, 7, 8, 9, 10, 13, 15)

(f) F(w, x, y, z) = ∑(0, 1, 2, 4, 5, 6, 7, 10, 15)

Chuyển từ F thành SOP và rút gọn chúng:

3.11 Convert the following Boolean function from a sum-of-products form to a simplified product-of-sums form.

F(x, y, z) = ∑(0, 1, 2, 5, 8, 10,

Chương 3: Logic Gates and Circuits

* Sử dụng các LOGIC GATES vẽ sơ đồ và tìm biểu thức rút gọn, tìm Karnaught maps

Chương 2: BOOLEAN ALGEBRA

*Định Lý Demorgan và vẽ mạch

2.6 Draw logic diagrams of the circuits that implement the original and simplified expressions in Problem 2.3.

2.7 Draw logic diagrams of the circuits that implement the original and simplified expressions in Problem 2.4.

2.8 Find the complement of F = wx + yz; then show that FF’ = 0 and F + F’ = 1.

2.9 Find the complement of the following expressions:

(a) xy’ + x’y

(b) (a + c).(a + b’).(a’ + b + c’)

(c) z + z’(v’w + xy)

2.10 Given the Boolean functions F1 and F2 , show that

(a) The Boolean function E = F1 + F2 contains the sum of the minterms of F1 and F2.

(b) The Boolean function G = F1F2 contains only the minterms that are common to F1 and F2.

2.16)The logical sum off all minterms of a Boolean function of n variables is 1.

a)Prove the previou statement for n = 3

b) Suggest a procedure for a general proof

*Cho mạch logic tìm biểu thức F

*Cho bảng chân trị tìm F